Lab Assignment 5

CMPE 146 Section 1

Professor Bhawandeep Singh Harsh

Andrew Bertch

Robert Trinh

Table of Contents

Exercise 1 – RC oscillator	3

Exercise 2 – Frequency measurement	4

Exercise 3 – LED control	8

Exercise 4 – Other oscillators	10

Appendix A – MCU Source Code	13

oscillate.c	13

measure_frequency.c	15

touch_LED.c	18

multiple_oscillators.c	23

# Exercise 1 – RC oscillator

In this exercise, we create a simple RC oscillator using one of the GPIO pins on the TI Launchpad. We can use the GPIO pin’s pull-up pull-down resistor for the resistance and the parasitic capacitances in the MCUs and boards wiring for the capacitance. By changing between either pull-down or pull-up on the pin, we can charge and discharge the capacitance creating an oscillator. This switch will occur when the pin reads high or low. Below is the console output from implementing the oscillator scheme above.

CORTEX_M0P: 0101010101010101010101010101010101010101010101010101010101010101010101010101010101010101010101010101

An alternative method could be achieved using the timer chip. We could decide a frequency that we want our oscillator to switch at. We could then poll the timer chip until the time elapses. A variable can then be switched from 0 or to 1 to keep track of the current state of the oscillator.

# Exercise 2 – Frequency measurement

In this exercise, we measure the frequency of the oscillator implemented in Exercise 1. We will also measure how touching the pin with a finger changes the frequency of the oscillator. Each cycle gets measured and printed to the console with a 1 second delay between measured cycles. The measured frequencies are shown below with tests 1-10 being touching the side of the pin with pressure, tests 11-20 touching the side of the pin with no pressure, tests 21-30 touching the top of the pin with pressure, tests 31-40 touching the top of the pin with no pressure, and tests 41-50 not touching the pin.

CORTEX_M0P: Test 1 starting now 58MHz

CORTEX_M0P: Test 2 starting now 59MHz

CORTEX_M0P: Test 3 starting now 60MHz

CORTEX_M0P: Test 4 starting now 59MHz

CORTEX_M0P: Test 5 starting now 63MHz

CORTEX_M0P: Test 6 starting now 66MHz

CORTEX_M0P: Test 7 starting now 58MHz

CORTEX_M0P: Test 8 starting now 64MHz

CORTEX_M0P: Test 9 starting now 66MHz

CORTEX_M0P: Test 10 starting now 66MHz

CORTEX_M0P: Test 11 starting now 37MHz

CORTEX_M0P: Test 12 starting now 45MHz

CORTEX_M0P: Test 13 starting now 49MHz

CORTEX_M0P: Test 14 starting now 48MHz

CORTEX_M0P: Test 15 starting now 49MHz

CORTEX_M0P: Test 16 starting now 53MHz

CORTEX_M0P: Test 17 starting now 49MHz

CORTEX_M0P: Test 18 starting now 52MHz

CORTEX_M0P: Test 19 starting now 53MHz

CORTEX_M0P: Test 20 starting now 54MHz

CORTEX_M0P: Test 21 starting now 37MHz

CORTEX_M0P: Test 22 starting now 45MHz

CORTEX_M0P: Test 23 starting now 42MHz

CORTEX_M0P: Test 24 starting now 48MHz

CORTEX_M0P: Test 25 starting now 48MHz

CORTEX_M0P: Test 26 starting now 44MHz

CORTEX_M0P: Test 27 starting now 46MHz

CORTEX_M0P: Test 28 starting now 47MHz

CORTEX_M0P: Test 29 starting now 43MHz

CORTEX_M0P: Test 30 starting now 34MHz

CORTEX_M0P: Test 31 starting now 27MHz

CORTEX_M0P: Test 32 starting now 27MHz

CORTEX_M0P: Test 33 starting now 27MHz

CORTEX_M0P: Test 34 starting now 28MHz

CORTEX_M0P: Test 35 starting now 28MHz

CORTEX_M0P: Test 36 starting now 28MHz

CORTEX_M0P: Test 37 starting now 29MHz

CORTEX_M0P: Test 38 starting now 29MHz

CORTEX_M0P: Test 39 starting now 28MHz

CORTEX_M0P: Test 40 starting now 29MHz

CORTEX_M0P: Test 41 starting now 13MHz

CORTEX_M0P: Test 42 starting now 13MHz

CORTEX_M0P: Test 43 starting now 13MHz

CORTEX_M0P: Test 44 starting now 13MHz

CORTEX_M0P: Test 45 starting now 13MHz

CORTEX_M0P: Test 46 starting now 13MHz

CORTEX_M0P: Test 47 starting now 13MHz

CORTEX_M0P: Test 48 starting now 13MHz

CORTEX_M0P: Test 49 starting now 13MHz

CORTEX_M0P: Test 50 starting now 13MHz

Something to note, the data collected was end time minus start time divided by 60. This is not the frequency and in my tables and graph was corrected by multiplying the data by 60 and then doing 1 divided by end minus start. Since f=1/T and end minus start is the period. The time was also measured in microseconds so in excel the formula was 1/(T*1e-6).

cod

Graph 2.1

Graph 2.1 plots the frequency measurements from the console. By plotting the this data it can be seen that touching the pin with more of our finger and with greater finger pressure the frequency of the oscillator goes down. That is, the oscillator is slower with more of a finger touching the pin. This is due to the capacitance of the circuit going up increasing the time constant of the discharge rate of the circuit.

The maximum speed of can be estimated by measuring a single charge to discharge cycle and cutting out all other extra lines of code. The implementation currently gets close but could see the frequency go as high as ~1400 Hz.

# Exercise 3 – LED control

In this exercise, we use capacitive touch control to turn on and off an LED. We make use of some of the code from the previous two exercises to measure and detect if a finger is touching to toggle a LED.

for(int i = 0; i < 5; i++)

{

printf("Test %d starting now ", test + 1);

start = DL_Timer_getTimerCount(TIMG12);

while (count < 20) {

if (!DL_GPIO_readPins(GPIOB, DL_GPIO_PIN_9) && prev != 0) {

count++;

prev = 0;

DL_GPIO_initPeripheralInputFunctionFeatures(IOMUX_PINCM26, IOMUX_PINCM26_PF_GPIOB_DIO09, DL_GPIO_INVERSION_DISABLE, DL_GPIO_RESISTOR_PULL_UP, DL_GPIO_HYSTERESIS_DISABLE, DL_GPIO_WAKEUP_DISABLE);

}

if (DL_GPIO_readPins(GPIOB, DL_GPIO_PIN_9) && prev != 1) {

count++;

prev = 1;

DL_GPIO_initPeripheralInputFunctionFeatures(IOMUX_PINCM26, IOMUX_PINCM26_PF_GPIOB_DIO09, DL_GPIO_INVERSION_DISABLE, DL_GPIO_RESISTOR_PULL_DOWN, DL_GPIO_HYSTERESIS_DISABLE, DL_GPIO_WAKEUP_DISABLE);

}

}

end = DL_Timer_getTimerCount(TIMG12);

count = 0;

prev = -1;

test++;

frequencySum += (end - start) / 60;

printf("%dhz\n", (end - start) / 60);

delay_cycles(32000000);

}

This code snippet does a measurement of the initial parameters to set a threshold value. As previously noted, the console outputs and variables are noted as being a frequency but are in fact the period. The console outputs that this generates as well as the calculated threshold value is shown below.

CORTEX_M0P: Test 1 starting now 15hz

CORTEX_M0P: Test 2 starting now 15hz

CORTEX_M0P: Test 3 starting now 15hz

CORTEX_M0P: Test 4 starting now 15hz

CORTEX_M0P: Test 5 starting now 15hz

CORTEX_M0P: Calibration threshold: 15hz

A video of this exercise functioning is included here.

# Exercise 4 – Other oscillators

In this exercise, we looked at oscillators on other GPIO pins. We run the same code from exercise 2 on 3 different pins and compared the measured period in microseconds divided by 60. This can be converted to frequency by multiplying by 60 and dividing 1 by the value. The console outputs are noted as being the frequency but are once again the period of the oscillator. The console outputs of this exercise are included below.

CORTEX_M0P: Test 1 starting now 14hz

CORTEX_M0P: Test 2 starting now 14hz

CORTEX_M0P: Test 3 starting now 14hz

CORTEX_M0P: Test 4 starting now 14hz

CORTEX_M0P: Test 5 starting now 14hz

CORTEX_M0P: Average Frequency PB9: 14hz

CORTEX_M0P: : Test 1 starting now 16hz

CORTEX_M0P: Test 2 starting now 16hz

CORTEX_M0P: Test 3 starting now 16hz

CORTEX_M0P: Test 4 starting now 16hz

CORTEX_M0P: Test 5 starting now 16hz

CORTEX_M0P: Average Frequency PB5: 16hz

CORTEX_M0P: : Test 1 starting now 17hz

CORTEX_M0P: Test 2 starting now 17hz

CORTEX_M0P: Test 3 starting now 17hz

CORTEX_M0P: Test 4 starting now 17hz

CORTEX_M0P: Test 5 starting now 17hz

CORTEX_M0P: Average Frequency PB11: 17hz

CORTEX_M0P: : Test 1 starting now 18hz

CORTEX_M0P: Test 2 starting now 18hz

CORTEX_M0P: Test 3 starting now 18hz

CORTEX_M0P: Test 4 starting now 18hz

CORTEX_M0P: Test 5 starting now 18hz

CORTEX_M0P: Average Frequency PB21: 18hz

The frequencies are likely different due to differences in the internal circuit capacitances for each pin. Each pin is connected to other chips and circuits on the board. This other devices and their supporting capacitors and traces can increase the internal capacitance and increase the charge discharge time of the RC circuit present on each pin assuming equal resistor values.

# Appendix A – MCU Source Code

## oscillate.c

#include "ti_msp_dl_config.h"

#include <stdio.h>

int count = 0;\

int prev = -1;

int main(void)

{

SYSCFG_DL_init();

DL_GPIO_enablePower(GPIOB);

DL_GPIO_initDigitalInput(IOMUX_PINCM26);

DL_GPIO_initPeripheralInputFunctionFeatures(IOMUX_PINCM26, IOMUX_PINCM26_PF_GPIOB_DIO09, DL_GPIO_INVERSION_DISABLE, DL_GPIO_RESISTOR_PULL_DOWN, DL_GPIO_HYSTERESIS_DISABLE, DL_GPIO_WAKEUP_DISABLE);

DL_GPIO_clearPins(GPIOB, DL_GPIO_PIN_9);

while (1) {

while (count < 100) {

if (!DL_GPIO_readPins(GPIOB, DL_GPIO_PIN_9) && prev != 0) {

count++;

prev = 0;

printf("0");

DL_GPIO_initPeripheralInputFunctionFeatures(IOMUX_PINCM26, IOMUX_PINCM26_PF_GPIOB_DIO09, DL_GPIO_INVERSION_DISABLE, DL_GPIO_RESISTOR_PULL_UP, DL_GPIO_HYSTERESIS_DISABLE, DL_GPIO_WAKEUP_DISABLE);

}

if (DL_GPIO_readPins(GPIOB, DL_GPIO_PIN_9) && prev != 1) {

count++;

prev = 1;

printf("1");

DL_GPIO_initPeripheralInputFunctionFeatures(IOMUX_PINCM26, IOMUX_PINCM26_PF_GPIOB_DIO09, DL_GPIO_INVERSION_DISABLE, DL_GPIO_RESISTOR_PULL_DOWN, DL_GPIO_HYSTERESIS_DISABLE, DL_GPIO_WAKEUP_DISABLE);

}

}

count = 0;

printf("\n");

}

}

## measure_frequency.c

#include "ti_msp_dl_config.h"

#include <stdio.h>

int count = 0;

int test = 0;

int prev = -1;

int start, end;

int main(void)

{

SYSCFG_DL_init();

DL_GPIO_enablePower(GPIOB);

DL_GPIO_initDigitalInput(IOMUX_PINCM26);

DL_GPIO_initPeripheralInputFunctionFeatures(IOMUX_PINCM26, IOMUX_PINCM26_PF_GPIOB_DIO09, DL_GPIO_INVERSION_DISABLE, DL_GPIO_RESISTOR_PULL_DOWN, DL_GPIO_HYSTERESIS_DISABLE, DL_GPIO_WAKEUP_DISABLE);

DL_GPIO_clearPins(GPIOB, DL_GPIO_PIN_9);

DL_Timer_enablePower(TIMG12);

DL_Timer_ClockConfig config;

config.clockSel = DL_TIMER_CLOCK_BUSCLK;

config.divideRatio = DL_TIMER_CLOCK_DIVIDE_1;

config.prescale = 0;

DL_Timer_setClockConfig(TIMG12, &config);

DL_Timer_TimerConfig timerConfig;

timerConfig.timerMode = DL_TIMER_TIMER_MODE_PERIODIC_UP;

timerConfig.period = -1;

timerConfig.startTimer = DL_TIMER_START;

timerConfig.genIntermInt = DL_TIMER_INTERM_INT_DISABLED;

timerConfig.counterVal = 0;

DL_Timer_initTimerMode(TIMG12, &timerConfig);

while (test < 50) {

printf("Test %d starting now ", test + 1);

start = DL_Timer_getTimerCount(TIMG12);

while (count < 20) {

if (!DL_GPIO_readPins(GPIOB, DL_GPIO_PIN_9) && prev != 0) {

count++;

prev = 0;

DL_GPIO_initPeripheralInputFunctionFeatures(IOMUX_PINCM26, IOMUX_PINCM26_PF_GPIOB_DIO09, DL_GPIO_INVERSION_DISABLE, DL_GPIO_RESISTOR_PULL_UP, DL_GPIO_HYSTERESIS_DISABLE, DL_GPIO_WAKEUP_DISABLE);

}

if (DL_GPIO_readPins(GPIOB, DL_GPIO_PIN_9) && prev != 1) {

count++;

prev = 1;

DL_GPIO_initPeripheralInputFunctionFeatures(IOMUX_PINCM26, IOMUX_PINCM26_PF_GPIOB_DIO09, DL_GPIO_INVERSION_DISABLE, DL_GPIO_RESISTOR_PULL_DOWN, DL_GPIO_HYSTERESIS_DISABLE, DL_GPIO_WAKEUP_DISABLE);

}

}

end = DL_Timer_getTimerCount(TIMG12);

count = 0;

prev = -1;

test++;

printf("%dhz\n", (end - start) / 60);

delay_cycles(32000000);

}

while (1) {

}

}

## touch_LED.c

#include "ti_msp_dl_config.h"

#include <stdio.h>

int count = 0;

int test = 0;

int prev = -1;

int start, end;

int frequencySum;

int thresholdParameter;

int main(void)

{

SYSCFG_DL_init();

DL_GPIO_enablePower(GPIOB);

DL_GPIO_initDigitalOutput(IOMUX_PINCM57); // PB26, red LED in LED2

DL_GPIO_initDigitalInput(IOMUX_PINCM26);

DL_GPIO_initPeripheralInputFunctionFeatures(IOMUX_PINCM26, IOMUX_PINCM26_PF_GPIOB_DIO09, DL_GPIO_INVERSION_DISABLE, DL_GPIO_RESISTOR_PULL_DOWN, DL_GPIO_HYSTERESIS_DISABLE, DL_GPIO_WAKEUP_DISABLE);

DL_GPIO_initDigitalInput(IOMUX_PINCM57); // PB26, red LED

DL_GPIO_enableOutput(GPIOB, DL_GPIO_PIN_26); // PB26, red LED

DL_GPIO_clearPins(GPIOB, DL_GPIO_PIN_9);

DL_GPIO_clearPins(GPIOB, DL_GPIO_PIN_26);

DL_Timer_enablePower(TIMG12);

DL_Timer_ClockConfig config;

config.clockSel = DL_TIMER_CLOCK_BUSCLK;

config.divideRatio = DL_TIMER_CLOCK_DIVIDE_1;

config.prescale = 0;

DL_Timer_setClockConfig(TIMG12, &config);

DL_Timer_TimerConfig timerConfig;

timerConfig.timerMode = DL_TIMER_TIMER_MODE_PERIODIC_UP;

timerConfig.period = -1;

timerConfig.startTimer = DL_TIMER_START;

timerConfig.genIntermInt = DL_TIMER_INTERM_INT_DISABLED;

timerConfig.counterVal = 0;

DL_Timer_initTimerMode(TIMG12, &timerConfig);

for(int i = 0; i < 5; i++)

{

printf("Test %d starting now ", test + 1);

start = DL_Timer_getTimerCount(TIMG12);

while (count < 20) {

if (!DL_GPIO_readPins(GPIOB, DL_GPIO_PIN_9) && prev != 0) {

count++;

prev = 0;

DL_GPIO_initPeripheralInputFunctionFeatures(IOMUX_PINCM26, IOMUX_PINCM26_PF_GPIOB_DIO09, DL_GPIO_INVERSION_DISABLE, DL_GPIO_RESISTOR_PULL_UP, DL_GPIO_HYSTERESIS_DISABLE, DL_GPIO_WAKEUP_DISABLE);

}

if (DL_GPIO_readPins(GPIOB, DL_GPIO_PIN_9) && prev != 1) {

count++;

prev = 1;

DL_GPIO_initPeripheralInputFunctionFeatures(IOMUX_PINCM26, IOMUX_PINCM26_PF_GPIOB_DIO09, DL_GPIO_INVERSION_DISABLE, DL_GPIO_RESISTOR_PULL_DOWN, DL_GPIO_HYSTERESIS_DISABLE, DL_GPIO_WAKEUP_DISABLE);

}

}

end = DL_Timer_getTimerCount(TIMG12);

count = 0;

prev = -1;

test++;

frequencySum += (end - start) / 60;

printf("%dhz\n", (end - start) / 60);

delay_cycles(32000000);

}

thresholdParameter = frequencySum / 5;

printf("Calibration threshold: %dhz\n", thresholdParameter);

int startControl, endControl;

int frequency;

bool isLEDOn = false;

while(1)

{

startControl = DL_Timer_getTimerCount(TIMG12);

while (count < 20) {

if (!DL_GPIO_readPins(GPIOB, DL_GPIO_PIN_9) && prev != 0) {

count++;

prev = 0;

DL_GPIO_initPeripheralInputFunctionFeatures(IOMUX_PINCM26, IOMUX_PINCM26_PF_GPIOB_DIO09, DL_GPIO_INVERSION_DISABLE, DL_GPIO_RESISTOR_PULL_UP, DL_GPIO_HYSTERESIS_DISABLE, DL_GPIO_WAKEUP_DISABLE);

}

if (DL_GPIO_readPins(GPIOB, DL_GPIO_PIN_9) && prev != 1) {

count++;

prev = 1;

DL_GPIO_initPeripheralInputFunctionFeatures(IOMUX_PINCM26, IOMUX_PINCM26_PF_GPIOB_DIO09, DL_GPIO_INVERSION_DISABLE, DL_GPIO_RESISTOR_PULL_DOWN, DL_GPIO_HYSTERESIS_DISABLE, DL_GPIO_WAKEUP_DISABLE);

}

}

endControl = DL_Timer_getTimerCount(TIMG12);

count = 0;

prev = -1;

frequency = (endControl - startControl) / 60;

if((frequency > thresholdParameter))

{

if(!isLEDOn)

{

DL_GPIO_setPins(GPIOB, DL_GPIO_PIN_26);

isLEDOn = true;

}

else if(isLEDOn)

{

DL_GPIO_clearPins(GPIOB, DL_GPIO_PIN_26);

isLEDOn = false;

}

}

delay_cycles(8000000);

}

}

## multiple_oscillators.c

#include "ti_msp_dl_config.h"

#include <stdio.h>

int countPB9 = 0;

int testPB9 = 0;

int prevPB9 = -1;

int countPB5 = 0;

int testPB5 = 0;

int prevPB5 = -1;

int countPB11 = 0;

int testPB11 = 0;

int prevPB11 = -1;

int countPB21 = 0;

int testPB21 = 0;

int prevPB21 = -1;

int startPB9, endPB9;

int startPB5, endPB5;

int startPB11, endPB11;

int startPB21, endPB21;

int frequencySumPB9, frequencySumPB5, frequencySumPB11, frequencySumPB21;

int averageFrequencyPB9, averageFrequencyPB5, averageFrequencyPB11, averageFrequencyPB21;

int main(void)

{

SYSCFG_DL_init();

DL_GPIO_enablePower(GPIOB);

DL_GPIO_initDigitalInput(IOMUX_PINCM26);

DL_GPIO_initDigitalInput(IOMUX_PINCM18);

DL_GPIO_initDigitalInput(IOMUX_PINCM28);

DL_GPIO_initDigitalInput(IOMUX_PINCM49);

DL_GPIO_initPeripheralInputFunctionFeatures(IOMUX_PINCM26, IOMUX_PINCM26_PF_GPIOB_DIO09, DL_GPIO_INVERSION_DISABLE, DL_GPIO_RESISTOR_PULL_DOWN, DL_GPIO_HYSTERESIS_DISABLE, DL_GPIO_WAKEUP_DISABLE);

DL_GPIO_initPeripheralInputFunctionFeatures(IOMUX_PINCM18, IOMUX_PINCM18_PF_GPIOB_DIO05, DL_GPIO_INVERSION_DISABLE, DL_GPIO_RESISTOR_PULL_DOWN, DL_GPIO_HYSTERESIS_DISABLE, DL_GPIO_WAKEUP_DISABLE);

DL_GPIO_initPeripheralInputFunctionFeatures(IOMUX_PINCM28, IOMUX_PINCM28_PF_GPIOB_DIO11, DL_GPIO_INVERSION_DISABLE, DL_GPIO_RESISTOR_PULL_DOWN, DL_GPIO_HYSTERESIS_DISABLE, DL_GPIO_WAKEUP_DISABLE);

DL_GPIO_initPeripheralInputFunctionFeatures(IOMUX_PINCM49, IOMUX_PINCM49_PF_GPIOB_DIO21, DL_GPIO_INVERSION_DISABLE, DL_GPIO_RESISTOR_PULL_DOWN, DL_GPIO_HYSTERESIS_DISABLE, DL_GPIO_WAKEUP_DISABLE);

DL_GPIO_clearPins(GPIOB, DL_GPIO_PIN_9);

DL_GPIO_clearPins(GPIOB, DL_GPIO_PIN_5);

DL_GPIO_clearPins(GPIOB, DL_GPIO_PIN_11);

DL_GPIO_clearPins(GPIOB, DL_GPIO_PIN_21);

DL_Timer_enablePower(TIMG12);

DL_Timer_ClockConfig config;

config.clockSel = DL_TIMER_CLOCK_BUSCLK;

config.divideRatio = DL_TIMER_CLOCK_DIVIDE_1;

config.prescale = 0;

DL_Timer_setClockConfig(TIMG12, &config);

DL_Timer_TimerConfig timerConfig;

timerConfig.timerMode = DL_TIMER_TIMER_MODE_PERIODIC_UP;

timerConfig.period = -1;

timerConfig.startTimer = DL_TIMER_START;

timerConfig.genIntermInt = DL_TIMER_INTERM_INT_DISABLED;

timerConfig.counterVal = 0;

DL_Timer_initTimerMode(TIMG12, &timerConfig);

while (testPB9 < 5) {

printf("Test %d starting now ", testPB9 + 1);

startPB9 = DL_Timer_getTimerCount(TIMG12);

while (countPB9 < 20) {

if (!DL_GPIO_readPins(GPIOB, DL_GPIO_PIN_9) && prevPB9 != 0) {

countPB9++;

prevPB9 = 0;

DL_GPIO_initPeripheralInputFunctionFeatures(IOMUX_PINCM26, IOMUX_PINCM26_PF_GPIOB_DIO09, DL_GPIO_INVERSION_DISABLE, DL_GPIO_RESISTOR_PULL_UP, DL_GPIO_HYSTERESIS_DISABLE, DL_GPIO_WAKEUP_DISABLE);

}

if (DL_GPIO_readPins(GPIOB, DL_GPIO_PIN_9) && prevPB9 != 1) {

countPB9++;

prevPB9 = 1;

DL_GPIO_initPeripheralInputFunctionFeatures(IOMUX_PINCM26, IOMUX_PINCM26_PF_GPIOB_DIO09, DL_GPIO_INVERSION_DISABLE, DL_GPIO_RESISTOR_PULL_DOWN, DL_GPIO_HYSTERESIS_DISABLE, DL_GPIO_WAKEUP_DISABLE);

}

}

endPB9 = DL_Timer_getTimerCount(TIMG12);

countPB9 = 0;

prevPB9 = -1;

testPB9++;

frequencySumPB9 += (endPB9 - startPB9) / 60;

printf("%dhz\n", (endPB9 - startPB9) / 60);

delay_cycles(32000000);

}

averageFrequencyPB9 =  frequencySumPB9 / 5;

printf("Average Frequency PB9: %dhz\n: ", averageFrequencyPB9);

while (testPB5 < 5) {

printf("Test %d starting now ", testPB5 + 1);

startPB5 = DL_Timer_getTimerCount(TIMG12);

while (countPB5 < 20) {

if (!DL_GPIO_readPins(GPIOB, DL_GPIO_PIN_5) && prevPB5 != 0) {

countPB5++;

prevPB5 = 0;

DL_GPIO_initPeripheralInputFunctionFeatures(IOMUX_PINCM18, IOMUX_PINCM18_PF_GPIOB_DIO05, DL_GPIO_INVERSION_DISABLE, DL_GPIO_RESISTOR_PULL_UP, DL_GPIO_HYSTERESIS_DISABLE, DL_GPIO_WAKEUP_DISABLE);

}

if (DL_GPIO_readPins(GPIOB, DL_GPIO_PIN_5) && prevPB5!= 1) {

countPB5++;

prevPB5 = 1;

DL_GPIO_initPeripheralInputFunctionFeatures(IOMUX_PINCM18, IOMUX_PINCM18_PF_GPIOB_DIO05, DL_GPIO_INVERSION_DISABLE, DL_GPIO_RESISTOR_PULL_DOWN, DL_GPIO_HYSTERESIS_DISABLE, DL_GPIO_WAKEUP_DISABLE);

}

}

endPB5 = DL_Timer_getTimerCount(TIMG12);

countPB5 = 0;

prevPB5 = -1;

testPB5++;

frequencySumPB5 += (endPB5 - startPB5) / 60;

printf("%dhz\n", (endPB5 - startPB5) / 60);

delay_cycles(32000000);

}

averageFrequencyPB5 =  frequencySumPB5 / 5;

printf("Average Frequency PB5: %dhz\n: ", averageFrequencyPB5);

while (testPB11 < 5) {

printf("Test %d starting now ", testPB11 + 1);

startPB11 = DL_Timer_getTimerCount(TIMG12);

while (countPB11 < 20) {

if (!DL_GPIO_readPins(GPIOB, DL_GPIO_PIN_11) && prevPB11 != 0) {

countPB11++;

prevPB11 = 0;

DL_GPIO_initPeripheralInputFunctionFeatures(IOMUX_PINCM28, IOMUX_PINCM28_PF_GPIOB_DIO11, DL_GPIO_INVERSION_DISABLE, DL_GPIO_RESISTOR_PULL_UP, DL_GPIO_HYSTERESIS_DISABLE, DL_GPIO_WAKEUP_DISABLE);

}

if (DL_GPIO_readPins(GPIOB, DL_GPIO_PIN_11) && prevPB11!= 1) {

countPB11++;

prevPB11 = 1;

DL_GPIO_initPeripheralInputFunctionFeatures(IOMUX_PINCM28, IOMUX_PINCM28_PF_GPIOB_DIO11, DL_GPIO_INVERSION_DISABLE, DL_GPIO_RESISTOR_PULL_DOWN, DL_GPIO_HYSTERESIS_DISABLE, DL_GPIO_WAKEUP_DISABLE);

}

}

endPB11 = DL_Timer_getTimerCount(TIMG12);

countPB11 = 0;

prevPB11 = -1;

testPB11++;

frequencySumPB11 += (endPB11 - startPB11) / 60;

printf("%dhz\n", (endPB11 - startPB11) / 60);

delay_cycles(32000000);

}

averageFrequencyPB11 =  frequencySumPB11 / 5;

printf("Average Frequency PB11: %dhz\n: ", averageFrequencyPB11);

while (testPB21 < 5) {

printf("Test %d starting now ", testPB21 + 1);

startPB21 = DL_Timer_getTimerCount(TIMG12);

while (countPB21 < 20) {

if (!DL_GPIO_readPins(GPIOB, DL_GPIO_PIN_21) && prevPB21 != 0) {

countPB21++;

prevPB21 = 0;

DL_GPIO_initPeripheralInputFunctionFeatures(IOMUX_PINCM49, IOMUX_PINCM49_PF_GPIOB_DIO21, DL_GPIO_INVERSION_DISABLE, DL_GPIO_RESISTOR_PULL_UP, DL_GPIO_HYSTERESIS_DISABLE, DL_GPIO_WAKEUP_DISABLE);

}

if (DL_GPIO_readPins(GPIOB, DL_GPIO_PIN_21) && prevPB21!= 1) {

countPB21++;

prevPB21 = 1;

DL_GPIO_initPeripheralInputFunctionFeatures(IOMUX_PINCM49, IOMUX_PINCM49_PF_GPIOB_DIO21, DL_GPIO_INVERSION_DISABLE, DL_GPIO_RESISTOR_PULL_DOWN, DL_GPIO_HYSTERESIS_DISABLE, DL_GPIO_WAKEUP_DISABLE);

}

}

endPB21 = DL_Timer_getTimerCount(TIMG12);

countPB21 = 0;

prevPB21 = -1;

testPB21++;

frequencySumPB21 += (endPB21 - startPB21) / 60;

printf("%dhz\n", (endPB21 - startPB21) / 60);

delay_cycles(32000000);

}

averageFrequencyPB21 =  frequencySumPB21 / 5;

printf("Average Frequency PB21: %dhz\n: ", averageFrequencyPB21);

while (1) {

}

}